ONR Technical Inspection Guide (TIG) LC18 – Radiological protection

# Introduction

- Many of the licence conditions (LCs) attached to the standard nuclear site licence require, or imply, that licensees should make arrangements to comply with regulatory obligations under the conditions. ONR inspects compliance with LCs, and the arrangements made under them, to judge the suitability of the arrangements made and the adequacy of their implementation. Most of the standard LCs are goal setting, and do not prescribe in detail what the licensees' arrangements should contain; this is the responsibility of the dutyholder who remains responsible for safety.

## Purpose

To support inspectors undertaking compliance inspection, ONR produces a suite of guides to assist inspectors to make regulatory judgements and decisions in relation to the adequacy of compliance, and the safety of activities on the site. This Technical Inspection Guide (TIG) has been prepared as a guide for ONR inspectors during which they judge the adequacy of LC compliance.

Seek advice and support of a radiological protection inspector to facilitate a LC18 inspection. This guidance should not be regarded as either comprehensive or mandatory.

# Licence Condition 18 – Radiological protection

18(1)	The licensee shall make and implement adequate arrangements for the assessment of the average effective dose (including any committed effective dose) to such class or classes of persons as may be specified in the aforesaid arrangements and the licensee shall forthwith notify ONR if the average effective dose equivalent to such class or classes of persons exceeds such level as ONR may specify.

18(2)	The licensee shall submit to ONR for approval such part or parts of the arrangements as ONR may specify.

- 18(3) The licensee shall ensure that once approved no alterations or amendment is made to the approved arrangements unless ONR has approved such alteration or amendment.

# Purpose of the Licence Condition

The purpose of this LC is to ensure that the licensee assesses the average effective dose equivalent to specified groups of employees and notifies ONR if these doses exceed a specified level. This purpose is in addition to the Ionising Radiation Regulations 2017 (IRR 17).

It should be noted that licensees of nuclear installations are required to comply with the IRR 17 [1], which has an Approved Code of Practice [2] which are the statutory requirements for the protection of persons against ionising radiation arising from work activities. There is a relevant TIG for IRR 17 inspections [3].

This LC implements Recommendation 3 of Sir Frank Layfield’s report on the Sizewell B Public Inquiry (published in 1987).

“I recommend that an average annual dose equivalent for the workforce at Sizewell B should be set as an operational investigation level. If the average dose equivalent, including any committed dose equivalent, were to exceed this level, the Nuclear Installations Inspectorate (NII), [now ONR], should investigate the circumstances with a view to ensuring that ALARP (as low as reasonably practicable) is being achieved.”

LC18(1) allows ONR the power to specify an average dose for notification to ONR. Upon receipt of a notification under LC18 the Inspector should inform their Head of Regulation and radiological protection specialist assessment effort should be requested to analyse the implications of the situation.

Other associated ONR guidance is the Technical Assessment Guide on Radiological Protection [4].

# Guidance on arrangements for Licence Condition 18

Arrangements should be provided to comply with all aspects of the LC18 requirements:

define the class or classes of persons and provide the means of identifying those for whom average doses should be kept.

ensure that arrangements approved under this LC, by ONR, can only be changed or amended with the approval of ONR. The person(s) responsible for ensuring compliance with this requirement should be identified in the arrangements.

include a requirement to notify ONR if the average dose to a class of persons exceeds the level specified by ONR, if appropriate.

Adequate records kept for the purpose of this condition are consistent with the arrangements made for LC6 (Documents, records, authorities and certificates) and LC25 (Operational records).

# Guidance on inspection of arrangements and their implementation

Inspection of these LC arrangements and their implementation will normally be undertaken as part of an IRR17 inspection.

Review the arrangements to establish validity, any changes made since the last review and that the identified responsible persons are correct. Note whether instructions, methods and quality assurance rules claimed in procedures have been followed and changes have been correctly incorporated and validated.

Confirm that the arrangements:

ensure dose assessment results are entered into a suitable dose record from which data for the average dose within classes can be determined.

enable the licensee to notify ONR if the average dose to a class of persons exceeds the level specified.

Confirm and sample that dose assessment results have been entered onto dose records, and that there are derived average doses for the identified classes.

Confirm that the identified classes, derived from criteria contained in the procedures, are still current. If changes have occurred, check that the reasons for changes are justified and have not been created to disguise groups that would otherwise have exceeded the specified level.

Where relevant, ONR may write to the licensee confirming that it will not be inspecting the requirement for a historical specification, until further notice. This may be relevant for example where no persons are currently employed within the classes identified by a licensee in their approved arrangements.

# References

| [1] | Ionising Radiations Regulations 2017, The Stationary Office, 2017. |
| --- | --- |
| [2] | Work with Ionising Radiation, Approved Code of Practice, L121, HSE Books 2018. |
| [3] | ONR, “NS-INSP-GD-054 Ionising Radiation Regulations 2017 Technical Inspection Guide.”. |
| [4] | ONR, “NS-TAST-GD-038 Radiological Protection Technical Assessment Guide (TAG)”. |

# Document control information

Approved by: Head of Profession – Operational Inspection

Issue: 7.1

Published: March 2026

Next scheduled review: October 2027

Document reference: NS-INSP-GD-018

Record reference: ONRHH-822789359-20463

Revision commentary

| Issue | Description of update(s) |
| --- | --- |
| 7 | Review with minor updates and revised ONR template. |
| 7.1 | Word count reduction and updated to the latest TIG template |