ONR Project assessment report PR-01941 – USA/9269/B(U) Revision 15 – ADR/RID validation of QSA Global Inc. Type B(U) package design for Model 650L

ONR Project assessment report

Project name: PR-01941

Report title: USA/9269/B(U) Revision 15 – ADR/RID validation of QSA Global Inc. Type B(U) package design for Model 650L

Dutyholder/Applicant: QSA Global Incorporated

Authored by:

Report issue no.: 1

Publication date:

Document ID: ONRW-2019369590-25566

Circulation list (latest issue)

| Organisation | Recipient | Date report sent |
| --- | --- | --- |

© Office for Nuclear Regulation, [2025]

For published documents, the electronic copy on the ONR website remains the most current publicly available version and copying or printing renders this document uncontrolled. If you wish to reuse this information visit www.onr.org.uk/copyright for details.

# Executive summary

This report presents the findings of the ONR assessment of the QSA Global Incorporated (QSA) application for validation of the Type B(U) package design for Model 650L, which is currently approved by the United States of America (US) Competent Authority (US Department of Transportation (DoT)) through certificate USA/9269/B(U), Revision 15.

The current US certificate states that the Model 650L package design meets the requirements of IAEA Regulations for the Safe Transport of Radioactive Material, 2018 Edition, No. SSR-6 (Revision 1).

The Model 650L source changer is a portable, depleted uranium shielded transport container used for transferring radiography source assemblies to and from industrial radiography exposure devices. The source changer is a Type B(U) transport container that is designed to safely contain radiography source assemblies during shipment and storage.

The assessment considered the following areas:

A review of the US DoT certification USA/9269/B(U), Revision 15 to confirm adequacy for validation and alignment with ADR requirements.

A completeness check of the Transposition Note, to confirm the Safety Analysis Report (SAR) paragraphs listed therein correspond to the required ADR paragraphs.

All areas were judged to have been satisfactorily met; one Regulatory (Level 4) Issue was raised.

Level 4 Issue (RI-12740):  QSA Global Incorporated ensure, before their next Model 650L validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

It is concluded that the package design meets the relevant regulatory requirements of ADR and RID.

# It is recommended that ONR validates US DoT package design approval USA/9269/B(U), Revision 15 by issuing GB validation certificate USA/9269/B(U) (Rev.0) with an expiration date of 30 November 2030.

Table 1: List of abbreviations.

| Term/Acronym | Description |
| --- | --- |
| ADR | Agreement concerning the International Carriage of Dangerous Goods by Road |
| CA | Competent Authority |
| CDG09 | The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 |
| DoT | Department of Transportation |
| GB | Great Britian |
| IAEA | The International Atomic Energy Agency |
| NRC | Nuclear Regulatory Commission |
| ONR | Office for Nuclear Regulation |
| PAR | Project Assessment Report |
| QSA | QSA Global Incorporated |
| RID | Regulation concerning the International Carriage of Dangerous Goods by Rail |
| SAR | Safety Analysis Report |
| SSR-6 | International Atomic Energy Agency Regulations for the Safe Transport of Radioactive Material |
| TN | Transposition Note |
| US | United States of America |

Table of contents

Executive summary	3

1.	Permission requested	6

2.	Background	6

2.1.	Package description	6

2.2.	Validation history	7

3.	Assessment and inspection work carried out by ONR in consideration of this request	7

3.1.	Assessment strategy and scope	7

3.2.	Assessment	8

3.3.	Review of US DoT Certification	8

3.4.	Transposition note	8

4.	Matters arising from ONR’s work	9

5.	Conclusions	9

6.	Recommendations	9

References	10

# Permission requested

QSA Global Incorporated (QSA, the applicant) has applied to ONR for validation of the Type B(U) package design for Model 650L (ref. [1]), which is currently approved by the United States of America Competent Authority (US Department of Transportation (DoT)) through certificate USA/9269/B(U), Revision 15 (ref. [2]).

This Project Assessment Report (PAR) presents the basis of a regulatory decision by the Office for Nuclear Regulation (ONR) as Great Britain (GB) competent authority (CA) for the transport of Class 7 (radioactive material) dangerous goods.  This statutory duty is given to ONR through ‘The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 (CDG09), which invokes the following modal regulations into GB-law:

Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) 2025 Edition.

Regulation concerning the International Carriage of Dangerous Goods by Rail (RID) 2025 Edition.

ADR and RID require that package designs granted unilateral approval by countries outside the ADR agreement, also need approval by the CA of a country that is a ‘Contracting Party’ to the Agreement, if packages are being transported in a country that is a ‘Contracting Party’. The 2025 editions of ADR and RID (for Class 7 dangerous goods) are based on the International Atomic Energy Agency (IAEA) Regulations for the Safe Transport of Radioactive Material, 2018 Edition, No. SSR-6, (Rev. 1).

The package design requirements for ADR and RID are the same, as is the paragraph numbering. Consequently, for the remainder of this report reference will only be made to ADR cognisant that any statements are also relevant to RID.

# Background

## Package description

The Model 650L source changer is a portable, depleted uranium shielded transport container used for transferring radiography source assemblies to and from industrial radiography exposure devices. The source changer is a Type B(U) transport container that is designed to safely contain radiography source assemblies during shipment and storage.

The Model 650L consists of a welded carbon or stainless steel cylindrical outer shell encasing a welded carbon or stainless steel rectangular inner shell. The inner shell contains a titanium “U” tube set in depleted uranium along with internal supports. The tube is crimped in the middle of the “U” to provide a positive stop for the source assembly. Additional shielding is provided by lead or tungsten positioned at various locations around the depleted uranium shield. The Model No. 650L has two source locking assemblies, mounted on the top cover plate, that are used to secure the radioactive special form source, iridium-192 or selenium-75, in a shielded position during transport. The packaging measures approximately 10-inches (254 mm) wide, 13.25-inches (337 mm) high and 8.25-inches (210 mm) deep. The maximum weight of the packaging is 90 pounds (41 kg).

Maximum activity of material per package:

Ir-192: 240 curies (8.9 TBq).

Se-75: 300 curies (11.1 TBq).

The maximum content mass is 0.08 pounds (36 g).

The full details of the package design are documented in the Safety Analysis Report (SAR, ref. [3]) submitted in support of the application. Additionally, a Transposition Note (TN, ref. [4]) has been submitted that provides a route map between the relevant ADR requirements and the section of the SAR where these are addressed.

## Validation history

We have validated all previous US DoT approvals for this package except for Revision 14. QSA submitted USA/9269/B(U), Revision 14 to ONR for validation in March 2025 but superseded this application in August 2025 (ref. [5]) with Revision 15 before ONR assessment of Revision 14 had commenced.

# Assessment and inspection work carried out by ONR in consideration of this request

## Assessment strategy and scope

In accordance with our regulatory permissioning strategy (PR-01941), we have carried out an assessment of QSA application for ADR validation of its Models 650L package design. This has taken due cognisance of ‘Guidance for Applications for UK Competent Authority Approval’, TRA-PER-GD-014 and ‘ONR Guidance Document, Transport Permissioning’ ONR-TCA-GD-001. Assessment of the application has focussed on the following matters:

A review of the US DoT certification USA/9269/B(U), Revision 15 to confirm adequacy for validation and alignment with ADR requirements.

A completeness check of the Transposition Note, to confirm the SAR paragraphs listed therein correspond to the required ADR paragraphs.

No inspection work was required as part of this validation.

## Assessment

A detailed note of our assessment was produced in support of the following key points (ref. [6]).

## Review of US DoT Certification

The US CA is identified as the US DoT with their certificate setting out general and specific conditions for the package.  The US DoT document references the US Nuclear Regulatory Commission (NRC) certificate of compliance (CoC) (ref. [7]) which details the package description and radioactive materials permitted to be transported in the package.

The US NRC provides a generic package content based on radionuclides and maximum activities. Recent assessment of another QSA Global package design, the 880 series, and recent ONR regulatory intelligence has identified examples of GB package consignors and operators using third party sources that are not consistent with those identified in the package operations and maintenance manual. Consequently, rather than endorsing the US DoT approval we have produced a separate validation certificate that stipulates that the authorised content for the package is for the specific assemblies detailed in the QSA operating manual (PR-01942).

We have adopted the same approach for the Model 650L package design considered in this report and recommend that the approved content is limited to those assemblies detailed in the relevant operating manual (ref. [8]).

## Transposition note

The TN submitted in support of the application correctly identifies the ADR obligations for a Type B(U) package and provides a route map showing how these requirements have been met through the claims and supporting evidence referenced in the SAR and QSA management system.  In the case of ageing management, a service bulletin for packages has been provided by QSA which has been attached to US CA certificate of approval. The service bulletins being developed from an ageing mechanisms evaluation (AME) undertaken by QSA.

# Matters arising from ONR’s work

In specifying the authorised radioactive contents of the package, a shortfall was highlighted in compliance of Paragraph 59 of ONR Guidance for Applications for UK Competent Authority Approval this stating ‘the Safety Case should be integrated.  The various parts of the application should be internally consistent’. This shortfall was also identified in our consideration of the application for validation of the QSA 880 series packages (PR-01942).

We have amended the existing Level 4 Issue (RI-12740) raised under PR-01942, to include the Model 650L package design. This requires that QSA ensure, before their next Model 650L validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

# Conclusions

Based on our assessment, I am satisfied that the package design meets the relevant regulatory requirements of ADR and RID.

The following Level 4 Issue has been raised (RI-12740):

- QSA Global Incorporated ensure, before their next Model 650L validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

# Recommendations

I recommend that ONR should validate US DoT package design approval USA/9269/B(U), Revision 15 by issuing GB validation certificate USA/9269/B(U) (Rev.0) with an expiration date of 30 November 2030 (ref. [9]).

# References

| [1] | QSA Letter, Application for Validation of USA/9269/B(U) Revision 14, 3 March 2025, ONRW-2019369590-18178. |
| --- | --- |
| [2] | US DoT Competent Authority Certification for a Type B(U) Package Design Cerrtificate, USA/9269/B(U), Revision 15, ONRW-2019369590-25307. |
| [3] | QSA Global Safety Analysis Report, Model 650L Type B(U) Transport Package, Revision 11, ONRW-2019369590-18194. |
| [4] | Transposition Note for Validation of USA/9269/B(U) Rev 15 to ADR 6.4.22.8(a) QSA Global Model 650L, ONRW-2019369590-25552. |
| [5] | QSA email, Revised expiry dates for US DoT approval, 26 August 2025, ONRW-2019369590-25564. |
| [6] | ONR Assessment Note for Model 650L, ONRW-2019369590-25562. |
| [7] | US NRC Certificate of Compliance for Radioactive Material Packages, USA/9269/B(U)-96, Revision 14, ONRW-2019369590-18181. |
| [8] | Model 650L Operations Manual, MAN-020, May 2021, ONRW-2019369590-25294. |
| [9] | GB Validation Certificate for USA/9269/B(U) Revision 15,ONRW-2019369590-25563. |