ONR Project assessment report PR-02018 – USA/6613/B(U) Revision 25 – ADR/RID validation of QSA Global Inc. Type B(U) package design for Model 702

ONR Project assessment report

Project name: PR-02018

Report title: USA/6613/B(U) Revision 25 – ADR/RID validation of QSA Global Inc. Type B(U) package design for Model 702

Dutyholder/Applicant: QSA Global Incorporated

Authored by:

Report issue no.: 1

Publication date: December 2025

Document ID: ONRW-2019369590-25610

Circulation list (latest issue)

| Organisation | Recipient | Date report sent |
| --- | --- | --- |

© Office for Nuclear Regulation, [2025]

For published documents, the electronic copy on the ONR website remains the most current publicly available version and copying or printing renders this document uncontrolled. If you wish to reuse this information visit www.onr.org.uk/copyright for details.

# Executive summary

This report presents the findings of the ONR assessment of the QSA Global Incorporated (QSA) application for validation of the Type B(U) package design for Model 702, which is currently approved by the United States of America (US) Competent Authority (US Department of Transportation (DoT)) through certificate USA/6613/B(U), Revision 25.

The current US certificate states that the Model 702 package design meets the requirements of IAEA Regulations for the Safe Transport of Radioactive Material, 2018 Edition, No. SSR-6 (Revision 1).

The Model 702 is designed as a transport package and storage container for Type B quantities of special form iridium-192, selenium-75, ytterbium-169 and caesium-137 radioactive material.

The assessment considered the following areas:

A review of the US DoT certification USA/6613/B(U), Revision 25 to confirm adequacy for validation and alignment with ADR requirements.

A completeness check of the Transposition Note, to confirm the Safety Analysis Report (SAR) paragraphs listed therein correspond to the required ADR paragraphs.

All areas were judged to have been satisfactorily met; two Regulatory (Level 4) Issue were raised.

RI-12740:  QSA Global Incorporated ensure, before their next Model 702 validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

- RI-12846: QSA Global Incorporated ensures that the safety important actions required to ensure safe configuration of the package are systematically identified as part of the safety case. The identified actions should be used to inform the design of the operating and maintenance instructions, to ensure that users of the package are provided with the information they need to use it safely.

It is concluded that the package design meets the relevant regulatory requirements of ADR and RID.

It is recommended that ONR validates US DoT package design approval USA/6613/B(U), Revision 25 by issuing GB validation certificate USA/6613/B(U) (Rev.0) with an expiration date of 29 February 2028.

Table 1: List of abbreviations.

| Term/Acronym | Description |
| --- | --- |
| ADR | Agreement concerning the International Carriage of Dangerous Goods by Road |
| CA | Competent Authority |
| CDG09 | The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 |
| DoT | Department of Transportation |
| GB | Great Britian |
| IAEA | The International Atomic Energy Agency |
| NRC | Nuclear Regulatory Commission |
| ONR | Office for Nuclear Regulation |
| PAR | Project Assessment Report |
| QSA | QSA Global Incorporated |
| RID | Regulation concerning the International Carriage of Dangerous Goods by Rail |
| SAR | Safety Analysis Report |
| SSR-6 | International Atomic Energy Agency Regulations for the Safe Transport of Radioactive Material |
| TN | Transposition Note |
| US | United States of America |

Table of contents

Executive summary	3

1.	Permission requested	6

2.	Background	6

2.1.	Package description	6

2.2.	Validation history	7

3.	Assessment and inspection work carried out by ONR in consideration of this request	7

3.1.	Assessment strategy and scope	7

3.2.	Assessment	8

3.3.	Review of US DoT Certification	8

3.4.	Transposition note	8

4.	Matters arising from ONR’s work	9

4.1.	Document configuration management	9

4.2.	Operations and maintenance manual	9

5.	Conclusions	9

6.	Recommendations	10

References	11

# Permission requested

QSA Global Incorporated (QSA, the applicant) has applied to ONR for validation of the Type B(U) package design for Model 702 (ref. [1]), which is currently approved by the United States of America Competent Authority (US Department of Transportation (DoT)) through certificate USA/6613/B(U), Revision 25 (ref. [2]).

This Project Assessment Report (PAR) presents the basis of a regulatory decision by the Office for Nuclear Regulation (ONR) as Great Britain (GB) competent authority (CA) for the transport of Class 7 (radioactive material) dangerous goods.  This statutory duty is given to ONR through ‘The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 (CDG09), which invokes the following modal regulations into GB-law:

Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) 2025 Edition.

Regulation concerning the International Carriage of Dangerous Goods by Rail (RID) 2025 Edition.

ADR and RID require that package designs granted unilateral approval by countries outside the ADR agreement, also need approval by the CA of a country that is a ‘Contracting Party’ to the Agreement, if packages are being transported in a country that is a ‘Contracting Party’. The 2025 editions of ADR and RID (for Class 7 dangerous goods) are based on the International Atomic Energy Agency (IAEA) Regulations for the Safe Transport of Radioactive Material, 2018 Edition, No. SSR-6, (Rev. 1).

The package design requirements for ADR and RID are the same, as is the paragraph numbering. Consequently, for the remainder of this report reference will only be made to ADR cognisant that any statements are also relevant to RID.

# Background

## Package description

The Model 702 is designed as a transport package and storage container for Type B quantities of special form iridium-192, selenium-75, ytterbium-169 and caesium-137 radioactive material.

The Model 702 is composed of a stainless steel cylinder shield cask assembly secured to a rectangular carbon steel skid and further covered by a carbon steel cage assembly that is bolted to the skid at each corner. The overall dimensions of the Model 702 are 19 ¾" x 21" x 19" (502 mm x 533 x 483 mm) and the maximum weight is 410 pounds (186 kg) including contents. The shield cask assembly contains a depleted uranium shield and a cover assembly sealed by a neoprene gasket. The cover assembly flange is anchored to the cask with six bolts.

The maximum activity of the material per package is

Cs-137 500 curies (18.5 TBq)

Se-75 10,000 curies (370 TBq)

Yb-169 10,000 curies (370 TBq)

Ir-192 15,000 curies (555 TBq)

The maximum content mass is 0.88 pounds (400 g).

The full details of the package design are documented in the Safety Analysis Report (SAR, ref. [3]) submitted in support of the application. Additionally, a Transposition Note (TN, ref. [4]) has been submitted that provides a route map between the relevant ADR requirements and the section of the SAR where these are addressed.

## Validation history

We have validated all previous US DoT approvals for this package except for Revision 24. QSA submitted USA/6613/B(U), Revision 24 to ONR for validation in February 2025 but superseded this application in September 2025 (ref. [5]) with Revision 25 before ONR assessment of Revision 24 had commenced.

# Assessment and inspection work carried out by ONR in consideration of this request

## Assessment strategy and scope

In accordance with our regulatory permissioning strategy (PR-02018), we have carried out an assessment of QSA application for ADR validation of its Model 702 package design. This has taken due cognisance of ‘Guidance for Applications for UK Competent Authority Approval’, TRA-PER-GD-014 and ‘ONR Guidance Document, Transport Permissioning’ ONR-TCA-GD-001. Assessment of the application has focussed on the following matters:

A review of the US DoT certification USA/6613/B(U), Revision 25 to confirm adequacy for validation and alignment with ADR requirements.

A completeness check of the Transposition Note, to confirm the SAR paragraphs listed therein correspond to the required ADR paragraphs.

No inspection work was required as part of this validation.

## Assessment

A detailed note of our assessment was produced in support of the following key points (ref. [6]).

## Review of US DoT Certification

The US CA is identified as the US DoT with their certificate setting out general and specific conditions for the package.  The US DoT document references the US Nuclear Regulatory Commission (NRC) certificate of compliance (CoC) (ref. [7]) which details the package description and radioactive materials permitted to be transported in the package.

The US NRC provides a generic package content based on radionuclides and maximum activities. Recent assessment of another QSA Global package design, the 880 series, and recent ONR regulatory intelligence has identified examples of GB package consignors and operators using third party sources that are not consistent with those identified in the package operations and maintenance manual. Consequently, rather than endorsing the US DoT approval we have produced a separate validation certificate that stipulates that the authorised content for the package is for the specific assemblies detailed in the QSA operating manual (PR-01942).

We have adopted the same approach for the Model 702 package design considered in this report and recommend that the approved content is limited to those assemblies detailed in the relevant operations manual (ref. [8]). This is discussed further in Section 4.2 of this report.

## Transposition note

The TN submitted in support of the application correctly identifies the ADR obligations for a Type B(U) package and provides a route map showing how these requirements have been met through the claims and supporting evidence referenced in the SAR and QSA management system.  In the case of ageing management, a service bulletin for packages has been provided by QSA which has been attached to the US CA certificate of approval.  The service bulletins being developed from an ageing mechanisms evaluation (AME) undertaken by QSA.

# Matters arising from ONR’s work

## Document configuration management

In specifying the authorised radioactive contents of the package, a shortfall was highlighted in compliance of Paragraph 59 of ONR Guidance for Applications for UK Competent Authority Approval this stating ‘the Safety Case should be integrated. The various parts of the application should be internally consistent’. This shortfall was also identified in our consideration of the application for validation of the QSA 880 series packages (PR-01942).

We have amended the existing Level 4 Issue (RI-12740) raised under PR-01942, to include the Model 702 package design. This requires that QSA ensure, before their next Model 702 validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

## Operations and maintenance manual

During our assessment we identified that QSA had not produced an operations and maintenance manual for the Model 702 package design. QSA advised that for this package they provided the operation and maintenance sections of the SAR to the package users. We advised that whilst these instructions were of a reasonable standard in identifying key tasks to be undertaken, they did not fully meet our expectations for compliance with relevant good practice. QSA has developed a draft manual. We consider that the standard of this manual is consistent with existing manuals for packages that we have already issued certificates for, however further opportunities for improvement and alignment with relevant good practice have been identified. These improvements will be tracked under a regulatory issue (RI-12846). Whilst the operations and maintenance manual remains a draft it is recommended that the tables of allowable content are reproduced in the validation certificate.

# Conclusions

Based on our assessment, I am satisfied that the package design meets the relevant regulatory requirements of ADR and RID.

The following Level 4 Issues have been raised:

- RI-12740: QSA Global Incorporated ensure, before their next Model 702 validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

- RI-12846: QSA Global Incorporated ensures that the safety important actions required to ensure safe configuration of the package are systematically identified as part of the safety case. The identified actions should be used to inform the design of the operating and maintenance instructions, to ensure that users of the package are provided with the information they need to use it safely.

# Recommendations

I recommend that ONR should validate US DoT package design approval USA/6613/B(U), Revision 25 by issuing GB validation certificate USA/6613/B(U) (Rev.0) with an expiration date of 29 February 2028 (ref. [9]).

# References

| [1] | QSA Letter, Application for Validation of USA/6613/B(U) Revision 24, 27 February 2025, ONRW-2019369590-18123. |
| --- | --- |
| [2] | US DoT Competent Authority Certification for a Type B(U) Package Design Cerrtificate, USA/6613/B(U), Revision 25, ONRW-2019369590-25301. |
| [3] | QSA Global Safety Analysis Report, Model 702 Type B(U) Transport Package, Revision 16, ONRW-2019369590-18124. |
| [4] | Transposition Note for Validation of USA/6613/B(U) Rev 25 to ADR 6.4.22.8(a) QSA Global Model 702, ONRW-2019369590-25300. |
| [5] | QSA email, Revised expiry dates for US DoT approval, 4 September 2025, ONRW-2019369590-25603. |
| [6] | ONR Assessment Note for Model 702, ONRW-2019369590-25609. |
| [7] | US NRC Certificate of Compliance for Radioactive Material Packages, USA/6613/B(U)-96, Revision 22, ONRW-2019369590-25301. |
| [8] | Model 702 Operations and Maintenance Manual, MAN-070 Draft, December 2025, ONRW-2019369590-25606. |
| [9] | GB Validation Certificate for USA/6613/B(U) Revision 25, ONRW-2019369590-25608. |