ONR Project assessment report PR-02019 – USA/9371/B(U) Revision 6 – ADR/RID validation of QSA Global Inc. Type B(U) package design for Model 360

ONR Project assessment report

Project name: PR-02019

Report title: USA/9371/B(U) Revision 6 – ADR/RID validation of QSA Global Inc. Type B(U) package design for Model 360

Dutyholder/Applicant: QSA Global Incorporated

Authored by:

Report issue no.: 1

Publication date: December 2025

Document ID: ONRW-2019369590-25581

Circulation list (latest issue)

| Organisation | Recipient | Date report sent |
| --- | --- | --- |

© Office for Nuclear Regulation, [2025]

For published documents, the electronic copy on the ONR website remains the most current publicly available version and copying or printing renders this document uncontrolled. If you wish to reuse this information visit www.onr.org.uk/copyright for details.

# Executive summary

This report presents the findings of the ONR assessment of the QSA Global Incorporated (QSA) application for validation of the Type B(U) package design for Model 360, which is currently approved by the United States of America (US) Competent Authority (US Department of Transportation (DoT)) through certificate USA/9371/B(U), Revision 6.

The current US certificate states that the Model 360 package design meets the requirements of IAEA Regulations for the Safe Transport of Radioactive Material, 2018 Edition, No. SSR-6 (Revision 1).

The Model 360 is a portable source changer, depleted uranium shielded, transport container used for transferring radiography source assemblies to and from industrial radiography exposure devices. The source changer is a Type B(U) transport container that is designed to safely contain radiography source assemblies during shipment and storage. The assessment considered the following areas:

A review of the US DoT certification USA/9371/B(U), Revision 6 to confirm adequacy for validation and alignment with ADR requirements.

A completeness check of the Transposition Note, to confirm the Safety Analysis Report (SAR) paragraphs listed therein correspond to the required ADR paragraphs.

All areas were judged to have been satisfactorily met; one Regulatory (Level 4) Issue was raised.

Level 4 Issue (RI-12740):  QSA Global Incorporated ensure, before their next Model 360 validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

It is concluded that the package design meets the relevant regulatory requirements of ADR and RID.

# It is recommended that ONR validates US DoT package design approval USA/9371/B(U), Revision 6 by issuing GB validation certificate USA/9371/B(U) (Rev.0) with an expiration date of 31 August 2028.

Table 1: List of abbreviations.

| Term/Acronym | Description |
| --- | --- |
| ADR | Agreement concerning the International Carriage of Dangerous Goods by Road |
| CA | Competent Authority |
| CDG09 | The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 |
| DoT | Department of Transportation |
| GB | Great Britian |
| IAEA | The International Atomic Energy Agency |
| NRC | Nuclear Regulatory Commission |
| ONR | Office for Nuclear Regulation |
| PAR | Project Assessment Report |
| QSA | QSA Global Incorporated |
| RID | Regulation concerning the International Carriage of Dangerous Goods by Rail |
| SAR | Safety Analysis Report |
| SSR-6 | International Atomic Energy Agency Regulations for the Safe Transport of Radioactive Material |
| TN | Transposition Note |
| US | United States of America |

Table of contents

Executive summary	3

1.	Permission requested	6

2.	Background	6

2.1.	Package description	6

2.2.	Validation history	8

3.	Assessment and inspection work carried out by ONR in consideration of this request	8

3.1.	Assessment strategy and scope	8

3.2.	Assessment	8

3.3.	Review of US DoT Certification	9

3.4.	Transposition note	9

4.	Matters arising from ONR’s work	9

5.	Conclusions	10

6.	Recommendations	10

References	11

# Permission requested

QSA Global Incorporated (QSA, the applicant) has applied to ONR for validation of the Type B(U) package design for Model 360 (ref. [1]), which is currently approved by the United States of America Competent Authority (US Department of Transportation (DoT)) through certificate USA/9371/B(U), Revision 6 (ref. [2]).

This Project Assessment Report (PAR) presents the basis of a regulatory decision by the Office for Nuclear Regulation (ONR) as Great Britain (GB) competent authority (CA) for the transport of Class 7 (radioactive material) dangerous goods. This statutory duty is given to ONR through ‘The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 (CDG09), which invokes the following modal regulations into GB-law:

Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) 2025 Edition.

Regulation concerning the International Carriage of Dangerous Goods by Rail (RID) 2025 Edition.

ADR and RID require that package designs granted unilateral approval by countries outside the ADR agreement, also need approval by the CA of a country that is a ‘Contracting Party’ to the Agreement, if packages are being transported in a country that is a ‘Contracting Party’. The 2025 editions of ADR and RID (for Class 7 dangerous goods) are based on the International Atomic Energy Agency (IAEA) Regulations for the Safe Transport of Radioactive Material, 2018 Edition, No. SSR-6, (Rev. 1).

The package design requirements for ADR and RID are the same, as is the paragraph numbering. Consequently, for the remainder of this report reference will only be made to ADR cognisant that any statements are also relevant to RID.

# Background

## Package description

The Model 360 is a portable source changer, depleted uranium shielded, transport container used for transferring radiography source assemblies to and from industrial radiography exposure devices. The source changer is a Type B(U) transport container that is designed to safely contain radiography source assemblies during shipment and storage.

The Model 360 consists of a family of five package configurations used to transport Ir-192, Yb-169, and Se-75 source assemblies that meet the requirements for special form material. These five packages are the Model Nos. 360-2, 360-4, 360-4W, 360-10, and 360-10W. The external dimensions of all models are identical and are approximately: 15 inches (38 cm) long, 12.5 inches (32 cm) wide, and 15.9 inches (40 cm) tall. The maximum weight of the package varies depending on the configuration and are as follows:

Model 360-2 110 lbs. (50 kg)

Model 360-4 130 lbs. (59 kg)

Model 360-4W 155 lbs. (70.3 kg)

Model 360-10 170 lbs. (77 kg)

Model 360-10W 180 lbs. (82 kg)

Maximum activity of material per package:

The maximum quantity of Se-75 and Yb-169 is 5,000 curies (185 TBq) for all variations of the Model 360 package.

For Ir-192, the values may depend on the location of the source activity in package shielding and are as follows:

| Model No. | - Activity Shield Location | - Maximum Package Capacity | - Maximum Number of Sources |
| --- | --- | --- | --- |
| 360-2 | Bottom | 300 curies (11.1 TBq) | - 2 |
|  | Top or Middle | 270 curies (9.99 TBq) |  |
| 360-4 | Bottom | 600 curies (22.2 TBq) | - 4 |
|  | Top of Middle | 540 curies (19.98 TBq) |  |
| 360-4W | Any | 260 curies (9.62 TBq) | - 4 |
| 360-10 | Any | 1,500 curies (55.5 TBq) | - 10 |
| 369-10W | Any | 170 curies (6.29 TBq) | - 10 |

The maximum content mass is:

0.3 lbs. (138 grams) for the Model 360-2 package.

0.6 lbs. (276 grams) for the Model 360-4 and 360-4W packages.

1.5 lbs. (690 grams) for the Model 360-10 and 360-10W packages.

The full details of the package design are documented in the Safety Analysis Report (SAR, ref. [3]) submitted in support of the application. Additionally, a Transposition Note (TN, ref. [4]) has been submitted that provides a route map between the relevant ADR requirements and the section of the SAR where these are addressed.

## Validation history

We have validated all previous US DoT approvals for this package except for Revision 5. QSA submitted USA/9371/B(U), Revision 5 to ONR for validation in February 2025 but superseded this application in September 2025 with Revision 6 before ONR assessment of Revision 5 had commenced.

# Assessment and inspection work carried out by ONR in consideration of this request

## Assessment strategy and scope

In accordance with our regulatory permissioning strategy (PR-2019), we have carried out an assessment of QSA application for ADR validation of its Models 360 package design. This has taken due cognisance of ‘Guidance for Applications for UK Competent Authority Approval’, TRA-PER-GD-014 and ‘ONR Guidance Document, Transport Permissioning’ ONR-TCA-GD-001. Assessment of the application has focussed on the following matters:

A review of the US DoT certification USA/9371/B(U), Revision 6 to confirm adequacy for validation and alignment with ADR requirements.

A completeness check of the Transposition Note, to confirm the SAR paragraphs listed therein correspond to the required ADR paragraphs.

No inspection work was required as part of this validation.

## Assessment

A detailed note of our assessment was produced in support of the following key points (ref. [5]).

## Review of US DoT Certification

The US CA is identified as the US DoT with their certificate setting out general and specific conditions for the package. The US DoT document references the US Nuclear Regulatory Commission (NRC) certificate of compliance (CoC) (ref. [6]) which details the package description and radioactive materials permitted to be transported in the package.

The US NRC provides a generic package content based on radionuclides and maximum activities. Recent assessment of another QSA Global package design, the 880 series, and recent ONR regulatory intelligence has identified examples of GB package consignors and operators using third party sources that are not consistent with those identified in the package operations and maintenance manual. Consequently, rather than endorsing the US DoT approval we have produced a separate validation certificate that stipulates that the authorised content for the package is for the specific assemblies detailed in the QSA operating manual (PR-01942).

We have adopted the same approach for the Model 360 package design considered in this report and recommend that the approved content is limited to those assemblies detailed in the relevant operations manual (ref. [7]).

## Transposition note

The TN submitted in support of the application correctly identifies the ADR obligations for a Type B(U) package and provides a route map showing how these requirements have been met through the claims and supporting evidence referenced in the SAR and QSA management system.  In the case of ageing management, a service bulletin for packages has been provided by QSA which has been attached to US CA certificate of approval. The service bulletins being developed from an ageing mechanisms evaluation (AME) undertaken by QSA.

# Matters arising from ONR’s work

In specifying the authorised radioactive contents of the package, a shortfall was highlighted in compliance of Paragraph 59 of ONR Guidance for Applications for UK Competent Authority Approval this stating ‘the Safety Case should be integrated.  The various parts of the application should be internally consistent’. This shortfall was also identified in our consideration of the application for validation of the QSA 880 series packages (PR-01942).

We have amended the existing Level 4 Issue (RI-12740) raised under PR-01942, to include the Model 360 package design. This requires that QSA ensure, before their next Model 360 validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

# Conclusions

Based on our assessment, I am satisfied that the package design meets the relevant regulatory requirements of ADR and RID.

The following Level 4 Issue has been raised (RI-12740):

- QSA Global Incorporated ensure, before their next Model 360 validation application, that a comprehensive configuration management exercise is undertaken, ensuring consistency throughout their safety analysis report and operational documentation.

# Recommendations

I recommend that ONR should validate US DoT package design approval USA/9371/B(U), Revision 6 by issuing GB validation certificate USA/9371/B(U) (Rev.0) with an expiration date of 31 August 2028 (ref. [8]).

# References

| [1] | QSA Letter, Application for Validation of USA/9371/B(U) Revision 5, 28 February 2025, ONRW-2019369590-18125. |
| --- | --- |
| [2] | US DoT Competent Authority Certification for a Type B(U) Package Design Cerrtificate, USA/9371/B(U), Revision 6, ONRW-2019369590-25313. |
| [3] | QSA Global Safety Analysis Report, Model 360 Series Type B(U) Transport Package, Revision 7 , ONRW-2019369590-18136. |
| [4] | Transposition Note for Validation of USA/9371/B(U) Rev 6 to ADR 6.4.22.8(a) QSA Global Model 360 Series, ONRW-2019369590-25551. |
| [5] | ONR Assessment Note for Model 360, ONRW-2019369590-25574. |
| [6] | US NRC Certificate of Compliance for Radioactive Material Packages, USA/9371/B(U)-96, Revision 4, ONRW-2019369590-25313. |
| [7] | Model 360 Operations Manual, MAN-040, May 2021, ONRW-2019369590-25295. |
| [8] | GB Validation Certificate for USA/9371/B(U) Revision 6, ONRW-2019369590-25576. |