ONR Project assessment report PR-02230 – Approval of Hinkley Point B emergency plan revision 17

ONR Project assessment report

Project name: PR-02230

Report title: Approval of Hinkley Point B emergency plan revision 17

Dutyholder/Applicant: EDF Energy Nuclear Generation Limited

Report issue no.: 1

Publication date:

Document ID: ONRW-2019369590-26929

© Office for Nuclear Regulation, 2026

For published documents, the electronic copy on the ONR website remains the most current publicly available version and copying or printing renders this document uncontrolled. If you wish to reuse this information visit www.onr.org.uk/copyright for details.

# Executive summary

Title

Approval of the Hinkley Point B nuclear power station site emergency plan revision 017.

Permission requested

EDF Energy Nuclear Generation Limited (the ‘licensee’) has requested our approval of an amendment to the site emergency plan for the Hinkley Point B nuclear power station in accordance with Licence Condition 11 (3).

Background

The nuclear site licence for Hinkley Point B requires the licensee to make and implement adequate arrangements for dealing with any accident or emergency arising on the site and their effects under licence condition 11. Since our previous approval, the licensee has completed defueling both of its reactors, transferred all the fuel off-site and completed a systematic fuel free verification process. This represents a significant reduction in the nuclear safety risk at Hinkley Point B.

The licensee has subsequently revised the emergency arrangements for Hinkley Point B to ensure that they are appropriate for the risk position of the station.

Assessment and inspection work carried out by ONR in consideration of this request

On the basis that we approved similar changes at Hunterston B in July 2025, our assessment and inspection has targeted the licensee’s justification of its post fuel free emergency response capability and observation of an emergency exercise demonstrating its implementation.

The Environment Agency have been consulted and have confirmed that they have no objections.

Matters arising from ONR's work

There are no outstanding matters arising from our assessment and inspection work.

Conclusions

Based on the evidence sampled we are satisfied that the licensee has:

Provided a suitable justification for the post fuel free emergency arrangements; and

Demonstrated an adequate emergency response capability.

To conclude, we are content that the proposed emergency arrangements are adequate for dealing with an accident or emergency on the site and we have not identified any issues that would prevent us approving the Hinkley Point B emergency plan.

Recommendation

ONR’s Director, Sellafield, Decommissioning, Fuel and Waste issues Licence Instrument 575 under LC 11(3) for Nuclear Site Licence 62C, approving revision 017 of the Hinkley Point B emergency plan.

Table 1: List of abbreviations.

| Term/Acronym | Description |
| --- | --- |
| HPB | Hinkley point B |
| INA | Independent nuclear assurance |
| LC | Licence condition |

Table of contents

Executive summary	4

1.	Permission requested	8

2.	Background	8

3.	Assessment and inspection work carried out by ONR in consideration of this request	9

3.1.	Justification of emergency response capability	9

3.2.	Observation of emergency response capability	10

3.3.	Independent nuclear assurance	10

3.4.	Engagement with other governmental agencies	11

4.	Matters arising from ONR’s work	11

5.	Conclusions	11

6.	Recommendations	12

References	12

# Permission requested

EDF Energy Nuclear Generation Limited (the licensee) has requested [1] our approval of revision 017 of the site emergency plan [2] for Hinkley Point B (HPB) power station under their arrangements for Licence Condition 11 (3).

# Background

The nuclear site license for HPB requires the licensee to make and implement adequate arrangements for dealing with any accident or emergency arising on the site and their effects under licence condition (LC) 11: emergency arrangements.

HPB’s emergency response arrangements are collectively defined in the site emergency plan and the emergency handbook. The plan describes the basic principles for dealing with any accident or emergency arising on the site, and the handbook details the procedures, equipment and personnel necessary to implement the response.

The site emergency plan is an approved document under LC 11(2), meaning that the licensee cannot alter or amend it without our approval under LC 11(3).

Since our approval of revision 016 of the emergency plan in January 2025, the licensee has completed defueling both of the reactors, transferred all of fuel off-site and completed a systematic fuel free verification process.

This represents a significant reduction in the nuclear safety risk at HPB and the licensee has subsequently reviewed its emergency arrangements to ensure that they are proportionate for the risk position of the station. The amended emergency plan introduces changes to the emergency response command structure, site response teams, emergency facilities and equipment. The plan establishes a core-on-site capability which is scalable dependant on the station operating condition, for example, normal working hours, extended working hours and silent hours.

The proposed arrangements represent progress towards implementing the contingency emergency arrangements which are consistent with those in place at other defueled reactor sites, and as such, designed to facilitate the subsequent transfer of HPB to Nuclear Restoration Services in preparation for decommissioning.

# Assessment and inspection work carried out by ONR in consideration of this request

We approved similar changes at Hunterston B (PR-02230) in July 2025, which was the first advanced gas-cooled reactor to complete fuel free verification and subsequently request a substantial change to the on-site emergency arrangements.

Based on operating experience from that assessment, we judged it proportionate to target our assessment for HPB on:

The differences between the Hunterston B and Hinkley Point B emergency arrangements.

Observation of an emergency exercise demonstrating the implementation of the proposed emergency response capability.

Confirmation that HPB are ready to implement the revised emergency arrangements.

The emergency plan also supports the licensee’s compliance with the Radiation Emergency Preparedness and Public Information Regulations. Our assessment only considered changes relevant to Regulation 10 (Operator’s emergency plan).

## Justification of emergency response capability

The licensee has developed the post fuel free emergency arrangements in accordance with its emergency planning policies and specifications for developing and maintaining emergency arrangements and hazard identification and risk evaluation for emergency preparedness and response.

Reference [3] presents the licensee’s justification for the post defueling emergency response capability. The licensee has used the fault and hazard schedules in the post defueling safety case, which we considered under PR-02173, to derive the proposed emergency arrangements. The licensee has considered radiological hazards, conventional hazards, security and other external events. Fleet wide operating experience has also been reviewed. Events that were screened out from the safety case based on design base criteria threshold were also considered.

We are satisfied with licensee’s safety case approach and content that the related fault analysis provides a suitable basis for developing the post fuel free emergency arrangements.

The licensee’s baseline emergency response requirements were derived for an operating station as part of the vision 2000 project, which defined specific roles, capabilities and minimum staffing levels. This has been used along with the safety case and associated fault analysis by the licensee’s subject matter experts to inform the development of the minimum task capability required for the post fuel free emergency response capability.

To validate the proposed arrangements, the licensee has assessed the capability against a representative range of emergency scenarios. The assessment did not identify any scenarios that would result in an inadequate response and found four scenarios which would provide the basis for stress testing.

We are satisfied that the licensee has applied a systematic process to develop a proportionate emergency response capability based on the identified hazards, consequences and safety cases.

Overall, we are satisfied that the licensee had adequately justified the proposed changes to the HPB emergency plan and have not identified any issues that would prevent our approval.

## Observation of emergency response capability

We observed a ‘proof of concept’ emergency exercise, [4] to gain confidence in the licensee’s proposed emergency arrangements.

We were satisfied that the exercise simulated a representative event based on the post fuel free hazard assessment and fleet operating experience. We judged that the exercise demonstrated that the proposed emergency arrangements were suitable for site to respond to a site incident. We did not observe any evidence that the proposed changes to the emergency plan would have any detrimental effect on the station’s emergency response.

## Independent nuclear assurance

The licensee’s independent nuclear assurance (INA) has assessed the proposed changes to HPB’s emergency response arrangements through its concurrence process. The assessment considered:

Governance and oversight

Change proposal, justification, and agreement / approval

Implementation strategy

Document review and updates

Training / awareness / authorisation (including exercises and drills)

Proof of concept and exercise development and delivery

Facilities and equipment changes

INA did not have any findings that raised cause for concern and consequently did not identify any areas for improvement or opportunities for improvement. INA has confirmed [5] that HPB has demonstrated readiness to implement the revised emergency arrangements and supports the application for a License Instrument.

## Engagement with other governmental agencies

Our arrangements require us to notify other competent regulatory authorities of our intention before issuing a licence instrument, to ensure that there are no specific objections that may compromise other regulatory requirements. The environment agency have confirmed [6] that they have no objections to us approving revision 017 of the HPB emergency plan.

# Matters arising from ONR’s work

There are no outstanding matters arising from our assessment and inspection carried out in consideration of this request.

# Conclusions

Based on the evidence sampled we are satisfied that the licensee has:

Provided a suitable justification for the post fuel free emergency response arrangements.

Demonstrated an adequate emergency response capability

To conclude, we are content that the proposed emergency arrangements are adequate for dealing with any accident or emergency arising on the site, and we have not identified any issues that would prevent us approving revision 017 of the HPB emergency plan.

# Recommendations

ONR’s Director, Sellafield, Decommissioning, Fuel and Waste issues Licence Instrument 575 under LC 11(3) for Nuclear Site Licence 62C, approving revision 017 of the Hinkley Point B emergency plan.

# References

| [1] | Letter, approval of site emergency plan – HINB/EHB/SEC/001 Rev 017,, NSL HPB 51480 R, 04 February 2026. ONRW-2019369590-26646. |
| --- | --- |
| [2] | Hinkley Point B Nuclear Power Station Post Fuel Free Verification Emergency Plan, HINB/EHB/SEC/01, Revision 017, February 2026. ONRW-2019369590-26645. |
| [3] | Hinkley Point B Post defueling emergency response capability, Justification Report - HINB/R/TQZ/063. ONRW-2019369590-26572. |
| [4] | ONR Contact Record, Hinkley Point B Proof of Concept Emergency Exercise CR-02114. |
| [5] | Independent nuclear assurance concurrence part B, Hinkley Point B changes to the emergency arrangements (LC11) post fuel free, SRD/REP/CON/HPB/028B, February 2026. ONRW-2019369590-26854. |
| [6] | Environment Agency - notice of no objection to approval of Hinkley Point B Emergency Plan - email dated 27 February 2026. ONRW-2019369590-26987. |